Supplementary Materials

S1. Description of the attributes (range, rate and weight) of the parameters of DRASTIC, GOD and SI methods (Aller et al., 1987; Barbulescu, 2020; Fannakh & Farsang, 2022; Kirlas et al., 2022; Rukmana et al., 2020)

| ID | Parameter or "Factor" | Range | Rate | Weight DRASTIC | Weight SI | Weight GOD |
| --- | --- | --- | --- | --- | --- | --- |
| D1 | D (m): Depth of water table | 0.0-1.5 | 10 |  |  |  |
| D2 |  | 1.5-3.0 | 9 |  |  |  |
| D3 |  | 3.0-4.5 | 8 |  |  |  |
| D4 |  | 4.5-9.0 | 7 |  |  |  |
| D5 |  | 9.0-15.0 | 5 | 5 | 0.186 | NA (a) |
| D6 |  | 15.0-22.5 | 3 |  |  |  |
| D7 |  | 22.5-30.0 | 2 |  |  |  |
| D8 |  | > 30 | 1 |  |  |  |
| R1 | R (mm): Net recharge | > 250 | 9 |  |  |  |
| R2 |  | 180-250 | 8 |  |  |  |
| R3 |  | 100-180 | 6 | 4 | 0.212 | NA |
| R4 |  | 50-100 | 3 |  |  |  |
| R5 |  | 0-50 | 1 |  |  |  |
| A1 | A: Aquifer media | Karst limestone | 10 |  |  |  |
| A2 |  | Basalt | 9 |  |  |  |
| A3 |  | Sand and gravel | 8 |  |  |  |
| A4 |  | Massive sandstone and limestone | 7 |  |  |  |
| A5 |  | Bedded sandstone and limestone | 6 | 3 | 0.259 | NA |
| A6 |  | Gralcial | 5 |  |  |  |
| A7 |  | Weathered metamorphic/igneous | 4 |  |  |  |
| A8 |  | Metamorphic/igneous | 3 |  |  |  |
| A9 |  | Massive shale | 2 |  |  |  |
| S1 | S: Soil media | Thin or absent, gravel | 10 |  |  |  |
| S2 |  | Sandstone and volcanic | 9 |  |  |  |
| S3 |  | Peat | 8 |  |  |  |
| S4 |  | Shrinking/aggregate clay/alluvium | 7 |  |  |  |
| S5 |  | Sand loam, schist, sand, karst volcanic | 6 | 2 | NA | NA |
| S6 |  | Loam | 5 |  |  |  |
| S7 |  | Silty loam | 4 |  |  |  |
| S8 |  | Clay loam | 3 |  |  |  |
| S9 |  | Muck acid, granitoid | 2 |  |  |  |
| S10 |  | Non shrink and non-aggregate clay | 1 |  |  |  |
| T1 | T (%): Topography | 0-2 | 10 |  |  |  |
| T2 |  | 2-3 | 9 |  |  |  |
| T3 |  | 3-4 | 8 |  |  |  |
| T4 |  | 4-5 | 7 |  |  |  |
| T5 |  | 5-6 | 6 |  |  |  |
| T6 |  | 6-10 | 5 | 1 | 0.121 | NA |
| T7 |  | 10-12 | 4 |  |  |  |
| T8 |  | 12-16 | 3 |  |  |  |
| T9 |  | 16-18 | 2 |  |  |  |
| T10 |  | >18 | 1 |  |  |  |
| I1 | I: Impact of the (unsaturated) vadose zone | Karst limestone | 10 |  |  |  |
| I2 |  | Basalt | 9 |  |  |  |
| I3 |  | Sand and gravel | 8 |  |  |  |
| I4 |  | Gravel, sand | 7 |  |  |  |
| I5 |  | Limestone, gravel, sand clay | 6 | 5 | NA | NA |
| I6 |  | Sandy silt | 5 |  |  |  |
| I7 |  | Metamorphic gravel and sandstone | 4 |  |  |  |
| I8 |  | Shale, silt, and clay | 3 |  |  |  |
| I9 |  | Silty clay | 2 |  |  |  |
| I10 |  | Confining layer, granite | 1 |  |  |  |
| C1 | C (m/ day): aquifer hydraulic conductivity | 0.04-4.1 | 1 |  |  |  |
| C2 |  | 4.1-12.3 | 2 |  |  |  |
| C3 |  | 12.3-28.7 | 4 | 3 | NA | NA |
| C4 |  | 28.7-41.0 | 6 |  |  |  |
| C5 |  | 41.0-82.0 | 8 |  |  |  |
| C6 |  | > 82.0 | 10 |  |  |  |
|  | LU: Land use |  |  |  |  |  |
| Lu1 | Agricultural areas | Irrigation schemes (annual corps), paddy fields | 90 |  |  |  |
| Lu2 |  | Permanent crops (orchards, vineyards) | 70 |  |  |  |
| Lu3 |  | Heterogeneous agricultural areas | 50 |  |  |  |
| Lu4 |  | Pasture and agro-forested areas | 50 |  |  |  |
| Lu5 | Artificial areas | Industrial waste discharges, landfills | 100 | NA | 0.222 | NA |
| Lu6 |  | Quarries, shipyards, open-air mines | 80 |  |  |  |
| Lu7 |  | Continuous urban areas, airports, harbors, (rail)roads, areas with industrial or commercial activities, laid out green spaces | 75 |  |  |  |
| Lu8 |  | Discontinuous urban areas | 70 |  |  |  |
| Lu9 | Natural area | Aquatic environments (salt marshes, salinas, intertidal zones) | 50 |  |  |  |
| Lu10 |  | Forests and semi-natural zones | 0 |  |  |  |
|  |  | Water bodies | 0 |  |  |  |
| G1 | G: groundwater occurrence | None | 0 |  |  |  |
| G2 |  | Overflowing | 0 |  |  |  |
| G3 |  | Confined | 0.2 |  |  |  |
| G4 |  | Semi-confined | 0.4 | NA | NA | NA |
| G5 |  | Unconfined (covered) | 0.6 |  |  |  |
| G6 |  | Unconfined | 0.7-1.0 |  |  |  |
| O1 | O: lithology of the vadose zone | Residual soils | 0.4 |  |  |  |
| O2 |  | Alluvial silt, loess, glacial till | 0.5 |  |  |  |
| O3 |  | Mudstones | 0.5 |  |  |  |
| O4 |  | Shales | 0.5 |  |  |  |
| O5 |  | Aeolian sands | 0.6 |  |  |  |
| O6 |  | Siltstones | 0.6 |  |  |  |
| O7 |  | Igneous/metamorphic formations | 0.6 |  |  |  |
| O8 |  | Volcanic tuff | 0.6-0.7 | NA | NA | NA |
| O9 |  | Alluvial and fluvio-glacial sands | 0.7 |  |  |  |
| O10 |  | Alluvial fan gravels | 0.8 |  |  |  |
| O11 |  | Sandstones | 0.7-0.8 |  |  |  |
| O12 |  | Recent volcanic lavas | 0.8 |  |  |  |
| O13 |  | Chalky limestone calcarenites | 0.9 |  |  |  |
| O14 |  | Calcretes+karst limestones | 0.9-1.0 |  |  |  |
| O15 |  | Karst limestone all depths | 0.9 |  |  |  |
| Dg1 | D (m): Depth of water table | < 5 | 0.9 |  |  |  |
| Dg2 |  | 5-20 | 0.8 |  |  |  |
| Dg3 |  | 20-50 | 0.7 | NA | NA | NA |
| Dg4 |  | 50-100 | 0.5 |  |  |  |
| Dg5 |  | > 100 | 0.4 |  |  |  |

(a) Non-Applicable

S2. Questionnaire used for the farmers in Boulbi.

| Family name: |  | First name : |  | Contact: |  |
| --- | --- | --- | --- | --- | --- |
| 1. What are the crops you currently cultivate, the areas and estimated yields? |  |  |  |  |  |
| Type | Area (ha) | Yield (tons/ha) | Planting date | Harvest date |  |
| 1.1 | …. | …. | …. | …. | …. |
| 1.2 | …. | …. | …. | …. | …. |
| 1.3 | …. | …. | …. | …. | …. |
| … | …. | …. | …. | …. | …. |
| 2. Do you use fertilizers? What quantities per campaign? NPK |  |  |  |  |  |
| Urea (kg/ha) | Organic matter (kg/ha) | Others |  |  |  |
| ….. | ….. | ….. | ….. | ….. | ….. |
| 3. Do you use plant protection products (pesticides)? If yes which ones and the quantity per campaign |  |  |  |  |  |
| Types | …. | …. | …. | …. |  |
| Quantities | …. | …. | …. | …. |  |
| 4. How many years have you been cultivating your land? |  |  |  |  |  |
| …. |  |  |  |  |  |
| 5. What treatments do you reserve for empty packaging of plant protection products? |  |  |  |  |  |
| …. Throw away ….Burn …. Reuse |  |  |  |  |  |
| 6. What is the origin of your fertilizer and pesticide? |  |  |  |  |  |
| Pesticides ….Authorized shop ….Street vendor ….Other |  |  |  |  |  |
| 7. What purpose do you use water from wells? |  |  |  |  |  |
| …. Irrigation only …. Domestic consumption …. Other |  |  |  |  |  |
| 8. What do you think about pesticide use? What do you recommend? |  |  |  |  |  |
| …. |  |  |  |  |  |